MULTI-NODE INFANT CRY DETECTION SYSTEM
WITH WEARABLE CAREGIVER ALERT

Official Hardware Bill of Materials (BOM) & Component Costing Schedule
——————————————————————————————————————————

1. Subsystem A: Cot Sensing Nodes (3 Dedicated Cot Units)

| Item | Component Name | Description & Specifications | Hardware Pinout / Notes | Qty | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | ESP32-C3-DevKitC-02 | RISC-V single-core @ 160MHz, 4MB Flash, 400KB SRAM, native 2.4GHz Wi-Fi/ESP-NOW | Node Core MCU (Cots 1, 2, 3) | 3 | 6,000 | 18,000 |
| A2 | INMP441 Digital MEMS Mic | Omnidirectional 24-bit I2S digital microphone module, bottom-port acoustic inlet | SCK: GPIO5, WS: GPIO6, SD: GPIO7, L/R: GND | 3 | 4000 | 12,000 |
| A3 | 100kΩ Resistors (0.25W, 1%) | Metal film precision resistors for 2:1 battery monitoring voltage divider | GPIO 1 (ADC1_CH1) divider network | 6 |  | 600 |
| A4 | 100nF (0.1µF) Ceramic Caps | Multilayer ceramic capacitors (MLCC) for ADC noise filtering & microphone decoupling | 1x across INMP441 VDD/GND, 1x on GPIO1 | 6 |  | 600 |
| A5 | 3mm / 5mm Red LED | High-efficiency Red LED for battery status signaling (solid, slow blink, fast blink) | GPIO 4 via 330Ω resistor (Active-HIGH) | 3 |  | 300 |
| A6 | 3mm / 5mm Blue LED | High-efficiency Blue LED for ESP-NOW link status; pulls GPIO 9 HIGH at boot | 3.3V -> 330Ω -> LED -> GPIO 9 (Active-LOW) | 3 |  | 300 |
| A7 | 330Ω Resistors (0.25W) | Current-limiting resistors for Red and Blue indicator LEDs | Red LED (GPIO 4) & Blue LED (GPIO 9) | 6 |  | 600 |
| A8 | TP4056 1S Li-Ion Charger | 1A constant-current/constant-voltage USB-C Li-Ion charging module with protection | Battery management (Cots 1, 2, 3) | 3 | 1,500 | 4,500 |
| A9 | 5V Step-Up Boost Converter | MT3608 micro DC-DC boost module (converts 3.7V Li-Po to 5.0V system rail) | Powers ESP32-C3 5V/VIN pin | 3 | 2,000 | 6,000 |
| A10 | 1S 3.7V 2000mAh Li-Po Cell | Rechargeable single-cell lithium-polymer battery with integrated PCM protection | Primary power source for cot nodes | 3 | 2,000 | 6,000 |
| A11 | 47µF - 100µF Electrolytic Caps | 25V radial electrolytic buffer capacitors for RF transmit transient suppression | Placed across 5V and GND boost rails | 3 |  | 1,000 |
| A12 | Solderless Breadboards (Half) | 400-tie point solderless half-size breadboards for clean workbench layout | Subsystem prototype mounting | 2 | 1,500 | 3,000 |

2. Subsystem B: Central Intelligence Hub (1 Unit)

| Item | Component Name | Description & Specifications | Hardware Pinout / Notes | Qty | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | ESP32-S3-DevKitC-1 N16R8 | Dual-Core Xtensa LX7 @ 240MHz, 16MB Quad-SPI Flash, 8MB Octal PSRAM | Central Edge-AI Hub (Strictly avoids GPIO 33-37) | 1 | 25,000 | 25,000 |
| B2 | 2.4" ILI9341 Color TFT Display | 240x320 SPI Color TFT with onboard 3.3V LDO; real-time cot GUI dashboard | CS: 10, MOSI: 11, SCK: 12, DC: 13, RESET: 14 | 1 | 20,000 | 20,000 |
| B3 | 100Ω Resistor (0.25W) | Current-limiting backlight ballast resistor for ILI9341 LED backlight | Placed between 3.3V rail and TFT LED pin | 1 | 300 | 300 |
| B4 | TP4056 1S Li-Ion Charger | 1A USB-C battery charging module with over-discharge and short protection | Central hub battery management | 1 | 1,500 | 1,500 |
| B5 | 5V Step-Up Boost Converter | MT3608 DC-DC step-up boost converter stepping 3.7V Li-Po up to 5.0V | Powers S3 5V/VIN and TFT VCC | 1 | 2,000 | 2,000 |
| B6 | 1S 3.7V 3000mAh Li-Ion / Li-Po | High-capacity rechargeable lithium cell for extended continuous hub runtime | Primary hub energy reservoir | 1 | 2,000 | 2,000 |
| B7 | 100µF 25V Electrolytic Cap | Radial decoupling capacitor across 5V boost converter output rail | Suppresses display and Wi-Fi current spikes | 1 | 100 | 100 |
| B8 | 100nF Ceramic Capacitor | High-frequency ceramic decoupling capacitor placed across ESP32-S3 3.3V rail | Power supply ripple rejection | 1 | 100 | 100 |
| B9 | Solderless Breadboard (Full) | 830-tie point solderless breadboard to mount ESP32-S3, TFT, and power modules | Central hub workbench mounting | 1 | 1,500 | 1,500 |

3. Subsystem C: Caregiver Wearable Alert Wristband (1 Unit)

| Item | Component Name | Description & Specifications | Hardware Pinout / Notes | Qty | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| C1 | ESP32-C3 SuperMini | Ultra-compact single-core RISC-V board with castellated pins and USB-C | Wearable MCU core (BLE Client) | 1 | 6000 | 6000 |
| C2 | 0.96" SSD1306 I2C OLED | 128x64 monochrome graphic OLED display; displays cot ID & alarm banner | SDA: GPIO 4, SCL: GPIO 5, VCC: 3.3V | 1 | 5,000 | 5,000 |
| C3 | IRLML2502TR (or AO3400A) | Logic-level N-Channel MOSFET in SOT-23 (Vth < 1.2V, Id > 4A, Rds(on) < 45mΩ) | Gate driven by GPIO 3 (switches motor GND) | 1 | 1,000 | 1,000 |
| C4 | 10mm Flat Coin ERM Motor | 3V ~60mA eccentric rotating mass coin vibration motor for silent tactile alerts | Connected between 3.3V and MOSFET Drain | 1 | 2,000 | 2,000 |
| C5 | 1N5819 Schottky Diode | Fast 40V 1A Schottky flyback diode; protects MOSFET from inductive kickback | Cathode to 3.3V, Anode to MOSFET Drain | 1 | 100 | 100 |
| C6 | 100Ω Resistor (0.25W) | Gate series damping resistor to suppress high-frequency switching oscillations | Between GPIO 3 and MOSFET Gate | 1 | 100 | 100 |
| C7 | 10kΩ Resistor (0.25W) | Gate pull-down resistor to guarantee motor remains completely OFF at boot | Between MOSFET Gate and GND | 1 | 100 | 100 |
| C8 | 1S 3.7V 500mAh Li-Po Cell | Compact ultra-lightweight Li-Po pouch battery for wristband form factor | Wearable wristband power source | 1 | 2,000 | 2,000 |
| C9 | TP4056 Micro/USB-C Board | Compact 1S Li-Po charging board with onboard battery protection | Wristband charging interface | 1 | 1,500 | 1,500 |
| C10 | Mini Breadboard / Perfboard | 170-tie point mini breadboard or compact prototyping perfboard | Wearable circuit prototyping | 1 | 1,500 | 1,500 |

4. Subsystem D: Hardware Prototyping Accessories & Workbench Consumables

| Item | Component Name | Description & Specifications | Hardware Pinout / Notes | Qty | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
| D1 | Premium Breadboard Jumpers (M-M) | 65-piece assorted male-to-male flexible solderless jumper wires (various lengths) | Inter-rail & IC breadboard connections | 2 packs |  | 2,400 |
| D2 | Breadboard Jumpers (M-F) | 40-pin female-to-male Dupont jumper ribbon cables for displays & breakouts | ILI9341 TFT & INMP441 mic connections | 2 packs |  | 2,400 |

5. Grand Cost Summary Table

| No. | Subsystem / Expenditure Category | Subtotal Cost (₦) |
| --- | --- | --- |
| 1 | Subsystem A: 3× Cot Sensing Nodes (MCUs, INMP441 Mics, LEDs, Li-Po Batteries, Boosters) | ₦52,900 |
| 2 | Subsystem B: 1× Central Intelligence Hub (ESP32-S3 N16R8, ILI9341 Color TFT, 3000mAh Battery) | ₦52,500 |
| 3 | Subsystem C: 1× Caregiver Wearable (ESP32-C3 SuperMini, SSD1306 OLED, MOSFET, ERM Motor) | ₦19,300 |
| 4 | Subsystem D: Prototyping Accessories, Cables, Connectors & Solder Consumables | ₦4,800 |
| 5 | Contingency / Miscellaneous Workshop Cost (~10%) | ₦12,950 |
| TOTAL | GRAND TOTAL ESTIMATED HARDWARE EXPENDITURE | ₦142,450 |